Automatic Passenger Counting System - FIT specification

TABLE OF CONTENTS

1	Introduction	6

1.1	Purpose	6

1.2	Applicability	6

1.3	Terms, Acronyms and Abbreviations	6

1.3.1	Acronyms and Abbreviations	6

1.3.2	Terms	11

1.4	Reference Documents	12

1.4.1	Contractual Documents	12

1.4.2	Project Documents	13

1.4.3	Standards and Regulations	15

1.5	Description of Changes From the Previous Revision	15

2	description of the equipment under test	16

3	location of test activities	17

3.1	Documentation	17

3.1.1	Test Plan	17

3.1.2	FIT Specification	17

3.1.3	SAT Specification	17

3.1.4	FIT	18

4	Prerequisites of the test	19

5	competent person	20

6	requirements under test	21

7	calibration/test equipment list	22

8	verification matrix	23

9	test environment	24

10	test methodology	25

11	test plan	26

12	test equipment	27

13	inspection sheets	28

TABLE OF FIGURES

Non è stata trovata alcuna voce dell'indice delle figure.

TABLE OF TABLES

Non è stata trovata alcuna voce dell'indice delle figure.

# Introduction

## Purpose

The purpose of this document is to provide the test procedures for the verification of the following test phase:

- Factory Integration Test (FIT).

## Applicability

The contents of the document apply to Factory Integration Tests for the verification of the integration of the functions provided by APCS.

  - Terms, Acronyms and Abbreviations

The following tables provide definitions for terms, acronyms, and abbreviations relevant to this document.

    - Terms

|  | Definition |
| --- | --- |
| Allocation | The assignment or budgeting of higher-level technical or nontechnical requirements among the lower-level partitioned functions for accomplishment. |
| Attribute | Descriptive information that provides important details about a module, requirement, discussion, or object. |
| Baseline | A specification or work product that has been formally reviewed and agreed on, which thereafter serves as the basis for further development, and which can be changed only through change control procedures. |
| Branch | The duplication of an object or set of objects under revision control to enable modification. |
| Configuration Control | An element of configuration management, consisting of the evaluation, coordination, approval or disapproval, and implementation of changes to configuration items after formal establishment of their baseline. |
| Customer Requirement | A technical or nontechnical requirement levied on Connect 6ix by the Contracting Authority. Customer Requirements are consolidated in the Customer Requirements Specification within the Requirements Register. |
| DecompoFITion | Transforming higher level requirements, either for the system or a subsystem, into successively lower-level detailed requirements. Higher-level requirements may be achieved by multiple subsystems or modules, each of which performs part of the functions. |
| Derivation | The changing or translation of a requirement through analysis into a form that is suitable for low-level analysis or design. |
| DOORS | Connect 6ix’s selected requirements database. |
| Flow-down | The systematic decompoFITion of requirements into allocated and derived requirements, appropriately assigned to low-level functional elements. |
| Generic Application (GA) | As part of the STS signaling product that is intended to be available for use by multiple projects. |
| Hazard Log | A living document applied to tracking all hazard analysis and hazard mitigation activities throughout the project lifecycle. |
| Interface Control Definition Document (ICDD) | The lowest level of interface between subsystems. The document identifies the technical solution to solve the interface. |
| Level of occupancy | A percentage value made by the following expression: ((number of passengers inside a car)/ (number of passengers in AW2 loading)) * 100. |
| Link | A link to an object in a module from another object elsewhere in the database. |
| Linkset | A division of information in each module that contains information about the links from one particular module to another. |
| Module | A set of requirements in DOORS. |
| Object | A self-contained piece of information in a module that can be arranged in a hierarchy and linked to show dependencies. |
| Requirement | (1) A condition or capability needed by a user to solve a problem or achieve an objective. (2) A condition or capability that must be met or possessed by a product, service, or product component to satisfy a supplier agreement, standard, specification, or other formally imposed documents. (3) A documented representation of a condition or capability as in (1) or (2). |
| Requirements Development | An iterative process to gather, review, and interpret technical contractual requirements, technical constraints, applicable laws and standards to produce system, subsystem requirements specifications. |
| Requirements Management | The management of the requirements received by or generated by the Project. |
| Requirements Register | The input requirements for the Project. The Customer Requirements Specification (CRS) contains the applicable CA’s requirements and comprises the largest portion of the Requirements Register. |
| Reliability | Ability of an item to perform a required function under given conditions for a given period of time. |
| Subcontractor | An external source that provides goods and/or services, including design responsibility and control. |
| System Element | A system element is a discrete part of a system that can be implemented to fulfil specified requirements. |
| Teamcenter | A corporate software tool that has several features including a formal verification process to release technical documents and is a secure repoFITory that manages configuration control of technical documents and parts. |
| Traceability | A discernible association between requirements and related requirements, implementations, verifications and validations. |
| Validation | Confirmation that the product or service, as provided (or as it will be provided), will fulfill its intended use. In other words, validation ensures that “you built the right thing”. |
| Verification | Confirmation that work products properly reflect the requirements specified for them. In other words, verification ensures that “you built it right”. |

    - Acronyms and Abbreviations

|  | Definition |
| --- | --- |
| AP | Access Point |
| APCS | Automatic Passenger Counting System |
| ATC | Automatic Train Control |
| ATP | Automatic Train Protection |
| ATS | Automatic Train Supervision |
| AW2 | Added Weight 2 |
| BI | Business Intelligence |
| BOCC | Backup Operations Control Centre |
| CA | Contracting Authority |
| C-APCS | Car Automatic Passenger Counting System |
| CBTC | Communication Based Train Control |
| CC | Carborne Controller |
| CCS | Communication & Control Centre |
| COMMS | Communications |
| CMMI | Capability Maturity Model Integration |
| CMP | Configuration Management Plan |
| CR | Change Request |
| CRS | Customer Requirements Specification |
| C/S | Codes and Standards |
| CU | Control Unit |
| CW | Civil Works |
| DCS | Data Communications System |
| DCU | Door Control Unit |
| DOORS | Dynamic Object-Oriented Requirements System |
| DWH | Data Warehouse |
| ENG | STS Engineering Department |
| FA | Final Acceptance |
| FAT | Factory Acceptance Test |
| FIT | Factory Integration Test |
| FCS | Fare Collection System |
| GA | Generic Application |
| GASC | Generic Application Safety Case |
| GPIO | General Purpose Input/Output |
| HMI | Human Machine Interface |
| HW | Hardware |
| HWRS | Hardware Requirement Specification |
| IAMS | Intelligent Asset Management System |
| ICDD | Interface Control Description Document |
| iFAT | Integrated Factory Acceptance Test |
| IL | Integration Layer |
| i-HMI | Integrated Human Machine Interface |
| iLAB | Integrated Laboratory |
| INCOSE | International Council on Systems Engineering |
| i-OCC | Integrated Operation Control Centre |
| IRIS | International Railway Industry Standard |
| ITR | Inspection Test Record |
| IXL | Interlocking |
| KPI | Key Performance Indicator |
| MMIS | Maintenance Management Information System |
| MSN | Multi Service Network |
| OBCU | On-Board Control Unit |
| OCC | Operation Control Centre |
| PBS | Product Breakdown Structure |
| PC | Personal Computer |
| PE | Project Engineer (Design Director) |
| PICO | Post Installation Check-Out |
| PIS-CU | Passenger Information System Control Unit |
| PoE | Power over Ethernet |
| PSD | Platform Screen Door |
| PVIS | Passenger Visualization Information System |
| QA | Quality Assurance |
| RAMS | Reliability, Availability, Maintainability and Safety |
| RFI | Request For Information |
| RFW | Request For Waiver |
| RM | Requirements Management |
| RMP | Requirements Management Plan |
| RS | Rolling Stock |
| RTC | Requirements Traceability Conformance |
| RTM | Requirements Traceability Matrix |
| RV | Revenue Vehicle |
| S&TC | Signalling & Train Control System |
| SA | Specific Application |
| SASC | Specific Application Safety Case |
| SAT | FITe Acceptance Test |
| SBS | System Breakdown Structure |
| SCADA | Supervisory Control and Data AcquiFITion System |
| SIL | Safety Integrity Level |
| FIT | System Integration Test |
| SDOT | System Demonstration Test |
| SyRS | System Requirements Specification |
| SSyRS | Subsystem Requirements Specification |
| STS | Hitachi Rail STS |
| SW | Software |
| SWRS | Software Requirements Specification |
| SV | System Validation |
| S&TCS | Signalling & Train Control System |
| TCMS | Train Control Monitoring System |
| T&C | Testing and Commissioning |
| TL | Test leader |
| TO | Traffic Operator |
| TP | Traction Power |
| V&V | Verification & Validation |
| WI | Work Item |
| WPL | Work Package Leader |

# description of the equipment under test

The testing activity will be conducted on the material/equipment listed in the following:

- C-APCS software

- C-APCS simulator orchestrator

- APCS backend software

- iOCC components

The verification covers:

- Interfaces between C-APCS and external sub-systems using simulators

- Interfaces between C-APCS and APCS backend

- Interfaces between APCS backend and iOCC

- Functions of APCS subsystem integrated in i-OCC

# location of test activities

The test will take place in factory at the following location:

•	Genoa Laboratory at Hitachi Rail facility

# Prerequisites of the test

The following conditions must be verified before the Test:

- C-APCS simulators orchestrator is available.

- APCS Back End is available.

- FAT Hardware and Software on vehicle must be performed and passed before to start the activity of integration.

- iOCC components are available.

# competent person

The Competent person are a set of people required for their expertise or experience to collaborate in order to return a result. A working group is listed based on subject matter expertise and availability to participate.

| Role | No Staff required | Proficiency / Competence |
| --- | --- | --- |
| APCS Design Test Lead | 1 | Yes/APCS Principal Engineer (WPL) |
| APCS Testing Engineer | 2 | Yes/APCS Engineer |

# calibration/test equipment list

The Laptop shall be used for the execution of the test. The web browser will be used to access the OBCU and iHMI. No other specific software is required. The latest version of Web browser shall be noted during the execution of the test.

# verification matrix

This chapter defines the requirements to be tested during the FIT.

| Test case ID | Test Case title | Requirements ID |
| --- | --- | --- |
| 5 | Vehicle crowding levels visualization on RTM section | C6-APCS-87 C6-APCS-72 C6-APCS-2 C6-APCS-74 C6-APCS-6 C6-APCS-7 C6-APCS-8 C6-APCS-9 C6-APCS-10 C6-APCS-11 C6-APCS-12 |
| 6 | “Empty” Level of crowding check on HMI and IL | C6-APCS-89 C6-APCS-82 C6-APCS-7 |
| 7 | “Light” Level of crowding check on HMI and IL | C6-APCS-88 C6-APCS-89 C6-APCS-8 |
| 8 | "Medium” Level of crowding check on HMI and IL | C6-APCS-83 C6-APCS-89 C6-APCS-9 |
| 9 | “High” Level of crowding check on HMI and IL | C6-APCS-84 C6-APCS-89 C6-APCS-10 |
| 10 | “Full” Level of crowding check on HMI and IL | C6-APCS-85 C6-APCS-89 C6-APCS-11 |
| 11 | Vehicle crowding levels update | C6-APCS-72 C6-APCS-12 |
| 12 | Station data visualization on RTM section | C6-APCS-3 |
| 13 | APCS-PVIS interface through IL | C6-APCS-89 C6-APCS-82 C6-APCS-88 C6-APCS-83 C6-APCS-84 C6-APCS-85 |
| 14 | FCS-APCS interface through IL | C6-APCS-3 |
| 15 | Collecting timestamps and statistics visualization on HMI | C6-APCS-4 |
| 16 | APCS-IAMS interface through IL | C6-APCS-75 |
| 17 | APCS-MMIS interface through IL | C6-APCS-1 |
| 18 | Diagnostics visualization on HMI | C6-APCS-90 C6-APCS-87 |
| 19 | ATS-APCS interface through IL | C6-APCS-1 |
| 20 | Interface test with PIS-CU | C6-APCS-2 |
| 21 | Interface test with TCMS | C6-APCS-2 |
| 22 | Software Integration Testing (SI) of i-OCC and APCS | C6-APCS-1 |
| 23 | Data retention test | C6-APCS-87 |

# test environment

# test methodology

The test results are defined as follow:

- PASS: Test is passed when all the steps are passed in accordance with the expected results (Test Pass / Fail Criteria).

- FAIL: Test is failed when one or more steps are not passed in accordance with the expected results (Test Pass / Fail Criteria).

- PASS WITH REMARKS: Test is passed with remarks if one or more steps have minor deviations to the expected results (Test Pass / Fail Criteria). A discrepancies list shall be included in the report. As for format, refer to the Test Plan[9] APPENDIX E DISCREPANCIES LIST FORM

# test plan

See Test Plan [9]

# test equipment

# ITR

The Inspection Test Record (ITR) shall be raised for System tesing the below Inspection sheets in Test Plan [9] Apendix E.

# test certificates

In this section for any test reported in the section 15, there are the form, information related to the test overview and the test procedure and final result.

# inspection sheets

  - Inspection sheet for FIT

| No | Activity Description | Test / Inspection Requirements And Criteria | Reference Documents (Drawing / Spec / MS / Standard / Code) | Frequency | Intervention Points |  |  | Verification Document No. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Subcon. | C6ix | CA |  |
| 3 | Interface test with ATS | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 4 | Interface test with MMIS | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 5 | Ridership data collection and visualization | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 6 | Storage and Displaying Verification | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 7 | Data update before arrival verification | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 8 | “Empty” Level of crowding check on HMI and IL | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 9 | “Light” Level of crowding check on HMI and IL | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 10 | "Medium” Level of crowding check on HMI and IL | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 11 | “High” Level of crowding check on HMI and IL | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 12 | “Full” Level of crowding check on HMI and IL | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 13 | FCS data receiving and displaying verification | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 14 | Collecting time stamps verification | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 15 | Interface Checking with Performance Reporting System | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 16 | Check the APCS accuracy | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 17 | Data update after departure verification | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 18 | Data retention test | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 19 | Fault Management Test | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 20 | Check APCS HMI functionalities for maintainer profile | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 21 | Check APCS HMI functionalities for OCC Operator profile | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 22 | Identification and authentication control (IAC) | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 23 | Use Control (UC) | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 24 | System integrity (SI) | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 25 | Data confidentiality (DC) | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 26 | Restricted data flow (RDF) | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 27 | Timely Responde to events (TRE) | As per Approved Test Procedure |  |  | - | W | W | Test Report |
| 28 | Resource availability (RA) | As per Approved Test Procedure |  |  | - | W | W | Test Report |

|  | Name | Title | Signature | Date |
| --- | --- | --- | --- | --- |
| Quality Assurance | Samy Sedfawi | Construction Quality Control Manager |  |  |
|  | Armin Amiressami | C6 - Construction Quality Manager |  |  |

W = Witness point:

- A work operation that is watched observed or visually examined by the Employer;

- The Contractor and the Employer must agree to witness points prior to the start of the work;

- The Contractor shall notify the Employer in advance of the operation to ensure the Employer can attend the required witness point.

H = Hold Point:

- A critical operation in which, by prior agreement, the Contractor shall notify the Employer a reasonable time in advance of the operation so that the item may be visually examined, tested or measured by the Employer to verify conformance with the Contract requirements;

- The Contractor may not proceed with work beyond such hold points except by written agreement of the Employer.

R = Review point:

- Review of the Contractor’s Inspection and Testing records by the Employer for correctness and completeness.

I = Inspection

- Responsible for carrying out inspection or test; includes sampling for testing.

S = Surveillance point:

- In process’ monitoring of the Contractor’s activities to the degree necessary for confidence that the work process/method is in compliance with the established criteria.

  - Test cases

    - Test Case ID: APCS_FIT_003

| ### TEST CASE ID: ### APCS_FIT_003 |  | ### SUB-SYSTEM: APCS |  |  |  |  | ### Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### Test Case Title: |  | ### Check Interface test with ATS |  |  |  |  |  |  |  |
| ### Category |  | ### Software |  |  |  |  |  |  |  |
| ### Ref. To Requirements: |  | ### C6-APCS-1 |  |  |  |  |  |  |  |
| ### Acceptance Criteria |  | ### PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| ### Test Objective |  | ### Check the interface between APCS back end and ATS, according to project documentation. |  |  |  |  |  |  |  |
| ### Ref. To Architecture |  | ### Architecture Reference |  |  |  |  |  |  |  |
| ### Reference Documents: |  | ### ICDD I-104B.OCC.APCS.14701 – Annex A ### S-212.21B - Automatic Passenger Counting System - Technical Specification ### S-212.21B - Automatic Passenger Counting System - HMI Specification |  |  |  |  |  |  |  |
| ### Tools / Configuration |  | ### Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | ### Equipments (if applicable) ### Model ### Serial Number |  |  |  |  |  |  |  |
|  |  | ### Calibration Date (if applicable) |  |  |  |  |  |  |  |
| ### Prerequisites: - SW FAT successfully completed - TL is placed in the OCC control room - TL can use the laptop to access the APCS back-end server - The OCC servers are powered on - The APCS Back-End is operational - The ATS is operational - The C-APCS simulator is operational |  |  |  |  |  |  |  |  |  |
| ### Scenario: ### According to project documentation, in the interface test between ATS and APCS backend through the IL (Integration Layer), a train is simulated to enter the service and move from the stabling area to the first station to verify that data can be correctly transmitted and received between the systems. |  |  |  |  |  |  |  |  |  |
| ### Location: ### OCC control room |  |  |  |  |  |  |  |  |  |
| ### Attachments: ### If any |  |  |  |  |  |  |  |  |  |
| ### Test Method |  |  |  |  |  |  |  |  |  |
| ### Test Process ### (Action) |  |  | ### Pass / Fail Criteria ### (Check) |  |  |  |  |  | ### Pass / Fail result |
| ### P1 | ### [TL] ### Login the APCS HMI |  | ### [TL] ### Check if the TL is logged in successfully |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P2 | ### [Sim] ### Run the APCS simulator to move the train from the stabling area to Station A. |  | ### [Sim] ### The train has arrived to A station |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P4 | ### [ATS] ### ATS sends the data to IL |  | ### [ATS] ### Check if the data is sent successfully |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P5 |  |  | ### [iOCC/IL] ### Check if the data is received successfully on IL |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P6 | ### [TL] ### The PVID-TID message is published on IL and read by APCS |  | ### [TL] ### Check if the APCS is subscribed to IL successfully. |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P7 | ### [TL] ### The PVID-MissionID message is published on IL and read by APCS |  | ### [TL] ### Check if the APCS is subscribed to IL successfully. |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P9 | ### [TL] ### Train Mode message is sent from the IL to APCS |  | ### [TL] ### Check if the data is received successfully. |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P10 | ### [TL] ### Revenue Service message is sent from the IL to APCS |  | ### [TL] ### Check if the data is received successfully. |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P11 | ### [TL] ### Coupling train |  | ### [TL] ### Check if the data is received successfully. |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P12 | ### [TL] ### Default Dwell Time (Headway mode) [OPTIONAL]: ### (Train Mode is Headway) |  | ### [TL] ### Check if the data is received successfully. |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P13 | ### [TL] ### Train Location [Optional]: |  | ### [TL] ### Check if the data is received successfully. |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P14 | ### [TL] ### Registry data |  | ### [TL] ### Check if the data is received successfully. |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P15 | ### [TL] ### Train composition |  | ### [TL] ### Check if the data is received successfully. |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### Resources: ### i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| ### TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| ### Date of Execution: |  |  |  |  |  |  |  |  |  |
| ### Tested by: |  |  |  |  | ### Witnessed By: |  |  |  |  |
| ### Signature: |  |  |  |  | ### Signature: |  |  |  |  |
| ### Results: |  | ### □ PASS |  | ### □ FAIL |  | ### □ PASS WITH REMARKS |  |  |  |
| ### Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_004

| ### TEST CASE ID: ### APCS_FIT_004 |  | ### SUB-SYSTEM: APCS |  |  |  |  | ### Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### Test Case Title: |  | ### Interface test with MMIS |  |  |  |  |  |  |  |
| ### Category |  | ### Software |  |  |  |  |  |  |  |
| ### Ref. To Requirements: |  | ### C6-APCS-1 |  |  |  |  |  |  |  |
| ### Acceptance Criteria |  | ### PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| ### Test Objective |  | ### Check the interface between APCS back end and MMIS, according to project documentation. |  |  |  |  |  |  |  |
| ### Ref. To Architecture |  | ### Architecture Reference |  |  |  |  |  |  |  |
| ### Reference Documents: |  | ### ICDD I-104B.OCC.APCS.14701 – Annex A ### S-212.21B - Automatic Passenger Counting System - User Manual ### S-212.21B - Automatic Passenger Counting System - Technical Specification |  |  |  |  |  |  |  |
| ### Tools / Configuration |  | ### Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | ### Equipments (if applicable) ### Model ### Serial Number |  |  |  |  |  |  |  |
|  |  | ### Calibration Date (if applicable) |  |  |  |  |  |  |  |
| ### Prerequisites: - SW FAT successfully completed - TL is placed in the OCC control room - TL can use the laptop to access the APCS back-end server - The OCC servers are powered on - The APCS Back-End is operational - The MMIS is operational - The C-APCS simulator is operational |  |  |  |  |  |  |  |  |  |
| ### Scenario: ### According to project documentation, in the interface test between MMIS and APCS backend through the IL (Integration Layer), simulate the diagnostic conditions of each APCS assets to be sent to MMIS to verify this interface. |  |  |  |  |  |  |  |  |  |
| ### Location: ### OCC control room |  |  |  |  |  |  |  |  |  |
| ### Attachments: ### If any |  |  |  |  |  |  |  |  |  |
| ### Test Method |  |  |  |  |  |  |  |  |  |
| ### Test Process ### (Action) |  |  | ### Pass / Fail Criteria ### (Check) |  |  |  |  |  | ### Pass / Fail result |
| ### P1 | ### [TL] ### Login the APCS HMI |  | ### [TL] ### Check if the TL is logged in successfully |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P2 | ### [TL] ### Simulate the following situation and send the corresponding message from APCS to IL: ### Train PVID - APCS on board unit - communication lost |  | ### [TL] ### Verify that the following code within the message is updated before and after simulating the corresponding situation ### 0 = communication available ### 1 = communication loss |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P3 | ### [TL] ### Simulate the following situation and send the corresponding message from APCS to IL: ### Train PVID - On-board unit Diagnostic High (>70%) |  | ### [TL] ### Verify that the following code within the message is updated before and after simulating the corresponding situation ### 0 = alarm off ### 1 = alarm on |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P4 | ### [TL] ### Simulate the following situation and send the corresponding message from APCS to IL: ### Train PVID - On-board unit Diagnostic Medium (>40% and <70%) |  | ### [TL] ### Verify that the following code within the message is updated before and after simulating the corresponding situation ### 0 = alarm off ### 1 = alarm on |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P5 | ### [TL] ### Simulate the following situation and send the corresponding message from APCS to IL: ### Train PVID - On-board unit Diagnostic Low (>10% and <40%) |  | ### [TL] ### Verify that the following code within the message is updated before and after simulating the corresponding situation ### 0 = alarm off ### 1 = alarm on |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P6 | ### [TL] ### Simulate the following situation and send the corresponding message from APCS to IL: ### Train PVID, APCS Sensor ID - Device Liveness (communication loss) |  | ### [TL] ### Verify that the following code within the message is updated before and after simulating the corresponding situation ### 0 = working status up ### 1 = working status down |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P7 | ### [TL] ### Simulate the following situation and send the corresponding message from APCS to IL: ### APCS Sofware Version (GLOBAL ID) |  | ### [TL] ### Verify that the following code within the message is sent after simulating the corresponding situation. |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### P8 | ### [TL] ### Simulate the following situation and send the corresponding message from APCS to IL: ### APCS Sofware Inconguity on Board Unit (Edge) |  | ### [TL] ### Verify that the following code within the message is sent after simulating the corresponding situation. ### 1 = failure |  |  |  |  |  | ### □ PASS ### □ FAIL ### □ PASS WITH REMARKS |
| ### Resources: ### i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| ### TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| ### Date of Execution: |  |  |  |  |  |  |  |  |  |
| ### Tested by: |  |  |  |  | ### Witnessed By: |  |  |  |  |
| ### Signature: |  |  |  |  | ### Signature: |  |  |  |  |
| ### Results: |  | ### □ PASS |  | ### □ FAIL |  | ### □ PASS WITH REMARKS |  |  |  |
| ### Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_005

| TEST CASE ID: APCS_FIT_005 |  | SUB-SYSTEM: APCS - HMI |  |  |  |  | Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Title: |  | Ridership data collection and visualization |  |  |  |  |  |  |  |
| Category |  | Software |  |  |  |  |  |  |  |
| Ref. To Requirements: |  | C6-APCS-1 C6-APCS-74 C6-APCS-2 |  |  |  |  |  |  |  |
| Acceptance Criteria |  | PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| Test Objective |  | Verify through APCS HMI visualization that, while train is simulated moving along the line, the APCS backend collects ridership data from the C-APCS: Train's ID, number of Passengers boarding and alighting each car at each station and ensure that these data are displayed on APCS HMI. |  |  |  |  |  |  |  |
| Ref. To Architecture |  | Refer to APCS-iOCC Architecture |  |  |  |  |  |  |  |
| Reference Documents: |  | 1414_RSM-RHR-CW-PWD-SPC-FCS0-00033_C03 - S-212.21A - Automatic Passenger Counting System - HMI Specification 1414_RSM-RHR-CW-PWD-SPC-FCS0-00041_C03 - S-212.21A - Automatic Passenger Counting system - Technical Specification |  |  |  |  |  |  |  |
| Tools / Configuration |  | Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | Equipments: Model Serial Number |  |  |  |  |  |  |  |
|  |  | Calibration Date (if applicable) |  |  |  |  |  |  |  |
| Prerequisites: - SW FAT successfully completed - TL is placed in the OCC control room - TL can use the laptop to access the APCS HMI - The OCC servers are powered on - The APCS Back-End is operational - The C-APCS simulator is operational |  |  |  |  |  |  |  |  |  |
| Scenario: OCC operator access to the HMI by credentials to check information related the train positioning, occupancy of the cars, number of the passengers received to the APCS HMI from the C-APCS. The test has been performed for each train simulated on the line to guarantee that the data exchange between OBCU – HMI it happens correctly. |  |  |  |  |  |  |  |  |  |
| Location: OCC |  |  |  |  |  |  |  |  |  |
| Attachments: If any |  |  |  |  |  |  |  |  |  |
| Test Method |  |  |  |  |  |  |  |  |  |
| Test Process (Action) |  |  | Pass / Fail Criteria (Check) |  |  |  |  |  | Pass / Fail result |
| P1 | OCC operator login to the APCS HMI by “Supervisor” profiles using: - Username - Password |  | Check that the access to I-HMI home page is correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P2 | Go into the section R.T.M (Reporting and Tracking Monitoring) |  | Check that are present on the dashboard: - Active trains on the line - Train ID - Last station visited by the train |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P3 | Active trains on the line: OCC operator recover the list of active trains on the line |  | The trains are: - Active on the line - In depot - Out of service |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P4 | OCC operator goes into the R.T.M section “Vehicles” |  | See on the screen the list only of the active trains on the line |  |  |  |  |  |  |
| P5 | See the dashboard with the status of the trains active on the line (according to the Fig.6 of the document HMI specification) |  | Need to check the status of the last icon on the right |  |  |  |  |  |  |
| P4 |  |  | Check that sees: - Train ID - Latest station ID - Total and divided occupancy per car - Each visited station on a specific journey |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P4 |  |  |  |  |  |  |  |  |  |
| Resources: i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| Date of Execution: |  |  |  |  |  |  |  |  |  |
| Tested by: |  |  |  |  | Witnessed By: |  |  |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  |
| Results: |  | □ PASS |  | □ FAIL |  | □ PASS WITH REMARKS |  |  |  |
| Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_006

| TEST CASE ID: APCS_FIT_006 |  | SUB-SYSTEM: APCS |  |  |  |  | Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Title: |  | Storage and display verification |  |  |  |  |  |  |  |
| Category |  | Software |  |  |  |  |  |  |  |
| Ref. To Requirements: |  | C6-APCS-87 |  |  |  |  |  |  |  |
| Acceptance Criteria |  | PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| Test Objective |  | Ensure that the APCS back-end system stores the data coming from C-APCS and displays it through statistical analysis using the BI on APCS HMI. |  |  |  |  |  |  |  |
| Ref. To Architecture |  | Refer to APCS-iOCC Architecture |  |  |  |  |  |  |  |
| Reference Documents: |  | 1414_RSM-RHR-CW-PWD-SPC-FCS0-00033_C03 - S-212.21A - Automatic Passenger Counting System - HMI Specification |  |  |  |  |  |  |  |
| Tools / Configuration |  | Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | Equipments: Model Serial Number |  |  |  |  |  |  |  |
|  |  | Calibration Date (if applicable) |  |  |  |  |  |  |  |
| Prerequisites: - SW FAT successfully completed - TL is placed in the OCC control room - TL can use the laptop to access the APCS HMI - The OCC servers are powered on - The APCS Back-End is operational - The C-APCS simulator is operational |  |  |  |  |  |  |  |  |  |
| Scenario: |  |  |  |  |  |  |  |  |  |
| Location: OCC – APCS HMI |  |  |  |  |  |  |  |  |  |
| Attachments: If any |  |  |  |  |  |  |  |  |  |
| Test Method |  |  |  |  |  |  |  |  |  |
| Test Process (Action) |  |  | Pass / Fail Criteria (Check) |  |  |  |  |  | Pass / Fail result |
| P1 | APCS reference person: Login to the APCS HMI by “Maintainer” profiles |  | Check that APCS person accesses to HMI home page correctly |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P2 | APCS reference person: Go to the section “statistics” |  | Check that APCS person sees the data received and collected by C-APCS system on the HMI correctly |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P3 | APCS reference person: Open the specific section to view additional information like: - Station Congestion Rate - Vehicle Ridership |  | Check that APCS person accesses to the two sections indicated and see the related graph correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P4 | APCS reference person: Check the other subsections related the primary sections: - Stations Congestion Rate - Vehicle Ridership |  | Check that APCS person sees the graph relating to the section he has chosen to check correctly, he will check: The graph related to: - Congestion Line - Trend Station by time - Carriage congestion |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| Resources: i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| Date of Execution: |  |  |  |  |  |  |  |  |  |
| Tested by: |  |  |  |  | Witnessed By: |  |  |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  |
| Results: |  | □ PASS |  | □ FAIL |  | □ PASS WITH REMARKS |  |  |  |
| Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_007

| TEST CASE ID: APCS_FIT_007 |  | SUB-SYSTEM: APCS |  |  |  |  | Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Title: |  | Data update before arrival verification |  |  |  |  |  |  |  |
| Category |  | Software |  |  |  |  |  |  |  |
| Ref. To Requirements: |  | C6-APCS-72 |  |  |  |  |  |  |  |
| Acceptance Criteria |  | PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| Test Objective |  | Check, before arrival at each station along the line, that the number of people that boarded and alighted each car at each previous station and the number of those who move from a car to another one during motion is updated on the APCS back-end. |  |  |  |  |  |  |  |
| Ref. To Architecture |  | Architecture Reference |  |  |  |  |  |  |  |
| Reference Documents: |  | HMI specification User manual |  |  |  |  |  |  |  |
| Tools / Configuration |  | Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | Equipments Model Serial Number |  |  |  |  |  |  |  |
|  |  | Calibration Date (if applicable) |  |  |  |  |  |  |  |
| Prerequisites: - SW FAT successfully completed - TL is placed in the OCC control room - TL can use the laptop to access the APCS HMI - The OCC servers are powered on - The APCS Back-End is operational - The C-APCS simulator is operational |  |  |  |  |  |  |  |  |  |
| Scenario: Starting from Exhibition Station, ensure that the congestion rate data is correctly updated before arriving at each station on the way to Science Centre Station. The passenger counting data for this test case will be inserted into OBCU and Backend using the test tool. |  |  |  |  |  |  |  |  |  |
| Location: Vehicle number OCC |  |  |  |  |  |  |  |  |  |
| Attachments: If any |  |  |  |  |  |  |  |  |  |
| Test Method |  |  |  |  |  |  |  |  |  |
| Test Process (Action) |  |  | Pass / Fail Criteria (Check) |  |  |  |  |  | Pass / Fail result |
| P1 | [TL]: Set a simulation scenario by sending counting data to APCS Back-End when doors have closed. |  | [TL]: Check the counting data arrive correctly on APCS Back-End. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P3 |  |  | [APCS TL]: Verify the counting data on each car of the train is updated before arrival at next station on HMI. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| Resources: i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| Date of Execution: |  |  |  |  |  |  |  |  |  |
| Tested by: |  |  |  |  | Witnessed By: |  |  |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  |
| Results: |  | □ PASS |  | □ FAIL |  | □ PASS WITH REMARKS |  |  |  |
| Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_008

| TEST CASE ID: APCS_FIT_008 |  | SUB-SYSTEM: APCS |  |  |  |  | Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Title: |  | Check the “Empty” Level of crowding check on HMI and IL |  |  |  |  |  |  |  |
| Category |  | Software |  |  |  |  |  |  |  |
| Ref. To Requirements: |  | C6-APCS-89 C6-APCS-82 C6-APCS-7 |  |  |  |  |  |  |  |
| Acceptance Criteria |  | PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| Test Objective |  | For a vehicle car in "empty" condition, ensure that the message sent to Integration Layer and the one provided to OCC Operator via APCS HMI about occupancy level, contains the string “empty” per the evaluated car. |  |  |  |  |  |  |  |
| Ref. To Architecture |  | Refer to APCS-iOCC Architecture |  |  |  |  |  |  |  |
| Reference Documents: |  | HMI specification ICDD IOCC |  |  |  |  |  |  |  |
| Tools / Configuration |  | Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | Equipments OBCU, sensor Model Serial Number |  |  |  |  |  |  |  |
|  |  | Calibration Date (if applicable) |  |  |  |  |  |  |  |
| Prerequisites: - SW FAT successfully completed - TL is placed in the OCC control room - TL can use the laptop to access the APCS HMI - The OCC servers are powered on - The APCS Back-End is operational - The C-APCS simulator is operational |  |  |  |  |  |  |  |  |  |
| Scenario: Check on APCS HMI the occupancy "empty" condition for the evaluated car of the train. |  |  |  |  |  |  |  |  |  |
| Location: Vehicle number |  |  |  |  |  |  |  |  |  |
| Attachments: If any |  |  |  |  |  |  |  |  |  |
| Test Method |  |  |  |  |  |  |  |  |  |
| Test Process (Action) |  |  | Pass / Fail Criteria (Check) |  |  |  |  |  | Pass / Fail result |
| P1 | APCS TL: Set a simulation scenario by setting counting data of a target train so it has 0% occupancy. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P2 | APCS TL: Login to the APCS HMI by OCC operator profiles |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P3 | APCS TL: Check the occupancy level 0% and the color on HMI is white for the occupancy of people for car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P4 | APCS TL: Check the occupancy level 0% and the color on HMI is white for the occupancy of people for car R1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P5 | Check the occupancy level 0% and the color on HMI is white for the occupancy of people for car S1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P6 | Check the occupancy level 0% and the color on HMI is white for the occupancy of people for car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P7 | IL TL: Check the data exchange related to the crowdedness for M1 car ID is 0% (empty level) have been published successfully from the APCS BE side and published on the IL. |  | Check the data has been correctly published on IL. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P8 | IL TL: Check the data exchange related to the crowdedness for R1 car ID is 0% (empty level) have been published successfully from the APCS BE side and published on the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P9 | Check the data exchange related to the crowdedness for S1 car ID is 0% (empty level) have been published successfully from the APCS BE side and published on the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P10 | Check the data exchange related to the crowdedness for M2 car ID is 0% (empty level) have been published successfully from the APCS BE side and published on the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P11 | APCS TL: Make the each car has 10% percent occupancy(15 persons for M1 and M2 and 17 for R1 and S1 cars). |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P12 | APCS TL: Login to the APCS HMI by OCC operator profiles |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P13 | APCS TL: Check the occupancy level =10% and the color on HMI is white for the occupancy of people for the car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P14 | APCS TL: Check the occupancy level =10% and the color on HMI is white for the occupancy of people for the car R1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P15 | APCS TL: Check the occupancy level =10% and the color on HMI is white for the occupancy of people for the car S1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P16 | APCS TL: Check the occupancy level =10% and the color on HMI is white for the occupancy of people for the car M2. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P17 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M1 car ID is 10% (empty level) have been published successfully from the BE side and arrived to the IL. |  | OCC/IL feedback that the message has been received correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P18 | IOCC/IL reference person: Check the data exchange related to the crowdedness for R1 car ID is 10% (empty level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P19 | IOCC/IL reference person: Check the data exchange related to the crowdedness for S1 car ID is 10% (empty level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P20 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M2 car ID is 10% (empty level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P21 | APCS reference person: Check the whole train Crowdedness level after each car has 10% of it’s capacity so the crowdedness for the whole train is 10 % and the number of people is 66 persons |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| Resources: i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| Date of Execution: |  |  |  |  |  |  |  |  |  |
| Tested by: |  |  |  |  | Witnessed By: |  |  |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  |
| Results: |  | □ PASS |  | □ FAIL |  | □ PASS WITH REMARKS |  |  |  |
| Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_009

| TEST CASE ID: APCS_FIT_007 |  | SUB-SYSTEM: APCS |  |  |  |  | Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Title: |  | Check the “light” Level of crowding check on HMI and IL |  |  |  |  |  |  |  |
| Category |  | Software |  |  |  |  |  |  |  |
| Ref. To Requirements: |  | C6-APCS-88 C6-APCS-89 C6-APCS-8 |  |  |  |  |  |  |  |
| Acceptance Criteria |  | PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| Test Objective |  | For a vehicle car in "light" condition, ensure that the message sent to Integration Layer and the one provided to OCC Operator via APCS HMI about occupancy level, contains the string “empty” per the evaluated car. |  |  |  |  |  |  |  |
| Ref. To Architecture |  | Architecture Reference figures 9 in the HMI spec IAMS figure in ICDD |  |  |  |  |  |  |  |
| Reference Documents: |  | HMI specification ICDD IOCC |  |  |  |  |  |  |  |
| Tools / Configuration |  | Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | Equipments OBCU, sensor Model Serial Number |  |  |  |  |  |  |  |
|  |  | Calibration Date (if applicable) |  |  |  |  |  |  |  |
| Prerequisites: - HW FAT, SW FAT, SW FIT, PICO, PRE-SAT and SAT tests are successfully completed. - Onboard MSN Network tests successfully completed. - OBCU is connected and fully tested successfully. - Sensors are connected to the MSN and fully tested that it sends the counting data to OBCU successfully. - APCS HM has been tested with all OCC operator profiles. - IL person to check the publication of IL. |  |  |  |  |  |  |  |  |  |
| Scenario: |  |  |  |  |  |  |  |  |  |
| Location: Vehicle number |  |  |  |  |  |  |  |  |  |
| Attachments: If any |  |  |  |  |  |  |  |  |  |
| Test Method |  |  |  |  |  |  |  |  |  |
| Test Process (Action) |  |  | Pass / Fail Criteria (Check) |  |  |  |  |  | Pass / Fail result |
| P1 | APCS reference person: Make the train has 11% percent occupancy(17 persons for M1 and M2 cars and 19 persons for car R1 and S1). |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P2 | APCS reference person: Login to the APCS HMI by OCC operator profiles |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P3 | APCS reference person: Check the occupancy level 11% and the color on HMI is green for the occupancy of people for car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P4 | APCS reference person: Check the occupancy level 11% and the color on HMI is green for the occupancy of people for car R1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P5 | Check the occupancy level 11% and the color on HMI is green for the occupancy of people for car S1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P6 | Check the occupancy level 11% and the color on HMI is green for the occupancy of people for car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P7 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M1 car ID is 11% (light level) have been published successfully from the BE side and arrived to the IL. |  | OCC/IL feedback that the message has been received correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P8 | IOCC/IL reference person: Check the data exchange related to the crowdedness for R1 car ID is 11% (light level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P9 | IOCC/IL reference person: Check the data exchange related to the crowdedness for S1 car ID is 11% (light level) have been published successfully from the BE side and arrived to the IL |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P10 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M2 car ID is 11% (light level) have been published successfully from the BE side and arrived to the IL |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P11 | APCS reference person: Make the each car has 30% percent occupancy(47 persons for M1 and M2 and 51 for R1 and S1 cars). |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P12 | APCS reference person: Login to the APCS HMI by OCC operator profiles |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P13 | APCS reference person: Check the occupancy level =30% and the color on HMI is green for the occupancy of people for the car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P14 | APCS reference person: Check the occupancy level =30% and the color on HMI is green for the occupancy of people for the car R1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P15 | APCS reference person: Check the occupancy level =30% and the color on HMI is green for the occupancy of people for the car S1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P16 | APCS reference person: Check the occupancy level =30% and the color on HMI is green for the occupancy of people for the car M2. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P17 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M1 car ID is 30% (light level) have been published successfully from the BE side and arrived to the IL. |  | OCC/IL feedback that the message has been received correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P18 | IOCC/IL reference person: Check the data exchange related to the crowdedness for R1 car ID is 30% (light level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P19 | IOCC/IL reference person: Check the data exchange related to the crowdedness for S1 car ID is 30% (light level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P20 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M2 car ID is 30% (light level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P21 | APCS reference person: Check the whole train Crowdedness level after each car has 30% of it’s capacity so the crowdedness for the whole train is 30 % and the number of people is 198 persons |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| Resources: i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| Date of Execution: |  |  |  |  |  |  |  |  |  |
| Tested by: |  |  |  |  | Witnessed By: |  |  |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  |
| Results: |  | □ PASS |  | □ FAIL |  | □ PASS WITH REMARKS |  |  |  |
| Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_010

| TEST CASE ID: APCS_FIT_007 |  | SUB-SYSTEM: APCS |  |  |  |  | Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Title: |  | Check the “Medium” Level of crowding check on HMI and IL |  |  |  |  |  |  |  |
| Category |  | Software |  |  |  |  |  |  |  |
| Ref. To Requirements: |  | C6-APCS-83 C6-APCS-89 C6-APCS-9 |  |  |  |  |  |  |  |
| Acceptance Criteria |  | PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| Test Objective |  | For a vehicle car in "light" condition, ensure that the message sent to Integration Layer and the one provided to OCC Operator via APCS HMI about occupancy level, contains the string “empty” per the evaluated car. |  |  |  |  |  |  |  |
| Ref. To Architecture |  | Architecture Reference figures 9 in the HMI spec IAMS figure in ICDD |  |  |  |  |  |  |  |
| Reference Documents: |  | HMI specification ICDD IOCC |  |  |  |  |  |  |  |
| Tools / Configuration |  | Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | Equipments OBCU, sensor Model Serial Number |  |  |  |  |  |  |  |
|  |  | Calibration Date (if applicable) |  |  |  |  |  |  |  |
| Prerequisites: - HW FAT, SW FAT, SW FIT, PICO, PRE-SAT and SAT tests are successfully completed. - Onboard MSN Network tests successfully completed. - OBCU is connected and fully tested successfully. - SW FAT successfully completed - TL is placed in the OCC control room - TL can use the laptop to access the APCS HMI - The OCC servers are powered on - The APCS Back-End is operational - The C-APCS simulator is operational |  |  |  |  |  |  |  |  |  |
| Scenario: |  |  |  |  |  |  |  |  |  |
| Location: Vehicle number |  |  |  |  |  |  |  |  |  |
| Attachments: If any |  |  |  |  |  |  |  |  |  |
| Test Method |  |  |  |  |  |  |  |  |  |
| Test Process (Action) |  |  | Pass / Fail Criteria (Check) |  |  |  |  |  | Pass / Fail result |
| P1 | APCS reference person: Make the train has 31% percent occupancy(49 persons for M1 and M2 cars and 54 persons for car R1 and S1). |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P2 | APCS reference person: Login to the APCS HMI by OCC operator profiles |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P3 | APCS reference person: Check the occupancy level 31% and the color on HMI is yellow for the occupancy of people for car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P4 | APCS reference person: Check the occupancy level 31% and the color on HMI is yellow for the occupancy of people for car R1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P5 | Check the occupancy level 31% and the color on HMI is white for the occupancy of people for car S1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P6 | Check the occupancy level 0% and the color on HMI is yellow for the occupancy of people for car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P7 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M1 car ID is 31% (medium level) have been published successfully from the BE side and arrived to the IL. |  | OCC/IL feedback that the message has been received correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P8 | IOCC/IL reference person: Check the data exchange related to the crowdedness for R1 car ID is 31% (medium level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P9 | IOCC/IL reference person: Check the data exchange related to the crowdedness for S1 car ID is 31% (medium level) have been published successfully from the BE side and arrived to the IL |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P10 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M2 car ID is 31% (medium level) have been published successfully from the BE side and arrived to the IL |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P11 | APCS reference person: Make the each car has 60% percent occupancy(94 persons for M1 and M2 and 104 for R1 and S1 cars). |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P12 | APCS reference person: Login to the APCS HMI by OCC operator profiles |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P13 | APCS reference person: Check the occupancy level =60% and the color on HMI is yellow for the occupancy of people for the car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P14 | APCS reference person: Check the occupancy level =60% and the color on HMI is yellow for the occupancy of people for the car R1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P15 | APCS reference person: Check the occupancy level =60% and the color on HMI is yellow for the occupancy of people for the car S1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P16 | APCS reference person: Check the occupancy level =60% and the color on HMI is yellow for the occupancy of people for the car M2. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P17 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M1 car ID is 60% (medium level) have been published successfully from the BE side and arrived to the IL. |  | OCC/IL feedback that the message has been received correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P18 | IOCC/IL reference person: Check the data exchange related to the crowdedness for R1 car ID is 60% (medium level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P19 | IOCC/IL reference person: Check the data exchange related to the crowdedness for S1 car ID is 60% (medium level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P20 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M2 car ID is 60% (medium level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P21 | APCS reference person: Check the whole train Crowdedness level after each car has 60% of its capacity so the crowdedness for the whole train is 60 % and the number of people is 396 persons |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| Resources: i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| Date of Execution: |  |  |  |  |  |  |  |  |  |
| Tested by: |  |  |  |  | Witnessed By: |  |  |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  |
| Results: |  | □ PASS |  | □ FAIL |  | □ PASS WITH REMARKS |  |  |  |
| Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_011

| TEST CASE ID: APCS_FIT_007 |  | SUB-SYSTEM: APCS |  |  |  |  | Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Title: |  | Check the “high” Level of crowding check on HMI and IL |  |  |  |  |  |  |  |
| Category |  | Software |  |  |  |  |  |  |  |
| Ref. To Requirements: |  | C6-APCS-84 C6-APCS-89 C6-APCS-10 |  |  |  |  |  |  |  |
| Acceptance Criteria |  | PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| Test Objective |  | For a vehicle car in "light" condition, ensure that the message sent to Integration Layer and the one provided to OCC Operator via APCS HMI about occupancy level, contains the string “empty” per the evaluated car. |  |  |  |  |  |  |  |
| Ref. To Architecture |  | Architecture Reference figures 9 in the HMI spec IAMS figure in ICDD |  |  |  |  |  |  |  |
| Reference Documents: |  | HMI specification ICDD IOCC |  |  |  |  |  |  |  |
| Tools / Configuration |  | Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | Equipments OBCU, sensor Model Serial Number |  |  |  |  |  |  |  |
|  |  | Calibration Date (if applicable) |  |  |  |  |  |  |  |
| Prerequisites: - SW FAT successfully completed - TL is placed in the OCC control room - TL can use the laptop to access the APCS HMI - The OCC servers are powered on - The APCS Back-End is operational - The C-APCS simulator is operational |  |  |  |  |  |  |  |  |  |
| Scenario: |  |  |  |  |  |  |  |  |  |
| Location: Vehicle number |  |  |  |  |  |  |  |  |  |
| Attachments: If any |  |  |  |  |  |  |  |  |  |
| Test Method |  |  |  |  |  |  |  |  |  |
| Test Process (Action) |  |  | Pass / Fail Criteria (Check) |  |  |  |  |  | Pass / Fail result |
| P1 | APCS reference person: Make the each car has 61% percent occupancy(96 persons for M1 and M2 and 106 for R1 and S1 cars). |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P2 | APCS reference person: Login to the APCS HMI by OCC operator profiles |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P3 | APCS reference person: Check the occupancy level 61% and the color on HMI is orange for the occupancy of people for car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P4 | APCS reference person: Check the occupancy level 61% and the color on HMI is orange for the occupancy of people for car R1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P5 | Check the occupancy level 61% and the color on HMI is orange for the occupancy of people for car S1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P6 | Check the occupancy level 61% and the color on HMI is orange for the occupancy of people for car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P7 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M1 car ID is 61% (high level) have been published successfully from the BE side and arrived to the IL. |  | OCC/IL feedback that the message has been received correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P8 | IOCC/IL reference person: Check the data exchange related to the crowdedness for R1 car ID is 61% (high level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P9 | Check the data exchange related to the crowdedness for S1 car ID is 61% (high level) have been published successfully from the BE side and arrived to the IL |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P10 | Check the data exchange related to the crowdedness for M2 car ID is 61% (high level) have been published successfully from the BE side and arrived to the IL |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P11 | APCS reference person: Make the each car has 95% percent occupancy(150 persons for M1 and M2 and 164 for R1 and S1 cars). |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P12 | APCS reference person: Login to the APCS HMI by OCC operator profiles |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P13 | APCS reference person: Check the occupancy level =95% and the color on HMI is orange for the occupancy of people for the car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P14 | APCS reference person: Check the occupancy level =95% and the color on HMI is white for the occupancy of people for the car R1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P15 | APCS reference person: Check the occupancy level =95% and the color on HMI is orange for the occupancy of people for the car S1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P16 | APCS reference person: Check the occupancy level =95% and the color on HMI is orange for the occupancy of people for the car M2. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P17 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M1 car ID is 95% (high level) have been published successfully from the BE side and arrived to the IL. |  | OCC/IL feedback that the message has been received correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P18 | IOCC/IL reference person: Check the data exchange related to the crowdedness for R1 car ID is 95% (high level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P19 | IOCC/IL reference person: Check the data exchange related to the crowdedness for S1 car ID is 95% (high level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P20 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M2 car ID is 95% (high level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P21 | APCS reference person: Check the whole train Crowdedness level after each car has 95% of it’s capacity so the crowdedness for the whole train is 95 % and the number of people is 627 persons |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| Resources: i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| Date of Execution: |  |  |  |  |  |  |  |  |  |
| Tested by: |  |  |  |  | Witnessed By: |  |  |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  |
| Results: |  | □ PASS |  | □ FAIL |  | □ PASS WITH REMARKS |  |  |  |
| Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_012

| TEST CASE ID: APCS_FIT_007 |  | SUB-SYSTEM: APCS |  |  |  |  | Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Title: |  | Check the “full” Level of crowding check on HMI and IL |  |  |  |  |  |  |  |
| Category |  | Software |  |  |  |  |  |  |  |
| Ref. To Requirements: |  | C6-APCS-85 C6-APCS-89 C6-APCS-11 |  |  |  |  |  |  |  |
| Acceptance Criteria |  | PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| Test Objective |  | For a vehicle car in "full" condition, ensure that the message sent to Integration Layer and the one provided to OCC Operator via APCS HMI about occupancy level, contains the string “full” per the evaluated car. |  |  |  |  |  |  |  |
| Ref. To Architecture |  | Architecture Reference figures 9 in the HMI spec IAMS figure in ICDD |  |  |  |  |  |  |  |
| Reference Documents: |  | HMI specification ICDD IOCC |  |  |  |  |  |  |  |
| Tools / Configuration |  | Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | Equipments OBCU, sensor Model Serial Number |  |  |  |  |  |  |  |
|  |  | Calibration Date (if applicable) |  |  |  |  |  |  |  |
| Prerequisites: - HW FAT, SW FAT, SW FIT, PICO, PRE-SAT and SAT tests are successfully completed. - Onboard MSN Network tests successfully completed. - OBCU is connected and fully tested successfully. - Sensors are connected to the MSN and fully tested that it sends the counting data to OBCU successfully. - APCS HM has been tested with all OCC operator profiles. - IL person to check the publication of IL. |  |  |  |  |  |  |  |  |  |
| Scenario: Check that the communication between Wayside MSN and theonboard OBCU works correctly. The test will be executed for all vehicles. |  |  |  |  |  |  |  |  |  |
| Location: Vehicle number |  |  |  |  |  |  |  |  |  |
| Attachments: If any |  |  |  |  |  |  |  |  |  |
| Test Method |  |  |  |  |  |  |  |  |  |
| Test Process (Action) |  |  | Pass / Fail Criteria (Check) |  |  |  |  |  | Pass / Fail result |
| P1 | APCS reference person: Make the each car has 95% percent occupancy(151 persons for M1 and M2 and 166 for R1 and S1 cars). |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P2 | APCS reference person: Login to the APCS HMI by OCC operator profiles |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P3 | APCS reference person: Check the occupancy level 96% and the color on HMI is red for the occupancy of people for car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P4 | APCS reference person: Check the occupancy level 96% and the color on HMI is red for the occupancy of people for car R1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P5 | Check the occupancy level 96% and the color on HMI is red for the occupancy of people for car S1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P6 | Check the occupancy level 96% and the color on HMI is red for the occupancy of people for car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P7 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M1 car ID is 96% (full level) have been published successfully from the BE side and arrived to the IL. |  | OCC/IL feedback that the message has been received correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P8 | IOCC/IL reference person: Check the data exchange related to the crowdedness for R1 car ID is 96% (full level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P9 | Check the data exchange related to the crowdedness for S1 car ID is 96% (full level) have been published successfully from the BE side and arrived to the IL |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P10 | Check the data exchange related to the crowdedness for M2 car ID is 96% (full level) have been published successfully from the BE side and arrived to the IL |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P11 | APCS reference person: Make the each car has 100% percent occupancy(157 persons for M1, 158 persons M2 and 173 for for R1 and S1 cars). |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P12 | APCS reference person: Login to the APCS HMI by OCC operator profiles |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P13 | APCS reference person: Check the occupancy level =100% and the color on HMI is red for the occupancy of people for the car M1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P14 | APCS reference person: Check the occupancy level =100% and the color on HMI is red for the occupancy of people for the car R1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P15 | APCS reference person: Check the occupancy level =100% and the color on HMI is red for the occupancy of people for the car S1. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P16 | APCS reference person: Check the occupancy level =100% and the color on HMI is red for the occupancy of people for the car M2. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P17 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M1 car ID is 100% (full level) have been published successfully from the BE side and arrived to the IL. |  | OCC/IL feedback that the message has been received correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P18 | IOCC/IL reference person: Check the data exchange related to the crowdedness for R1 car ID is 100% (full level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P19 | IOCC/IL reference person: Check the data exchange related to the crowdedness for S1 car ID is 100% (full level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P20 | IOCC/IL reference person: Check the data exchange related to the crowdedness for M2 car ID is 100% (full level) have been published successfully from the BE side and arrived to the IL. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P21 | APCS reference person: Check the whole train Crowdedness level after each car has 100% of it’s capacity so the crowdedness for the whole train is 100 % and the number of people is 661 persons |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| Resources: i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| Date of Execution: |  |  |  |  |  |  |  |  |  |
| Tested by: |  |  |  |  | Witnessed By: |  |  |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  |
| Results: |  | □ PASS |  | □ FAIL |  | □ PASS WITH REMARKS |  |  |  |
| Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_020

| TEST CASE ID: APCS_FIT_020 |  | SUB-SYSTEM: APCS |  |  |  |  | Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Title: |  | Check APCS HMI functionalities for maintainer profile |  |  |  |  |  |  |  |
| Category |  | software |  |  |  |  |  |  |  |
| Ref. To Requirements: |  | C6-APCS-90 C6-APCS-87 |  |  |  |  |  |  |  |
| Acceptance Criteria |  | PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| Test Objective |  | Check that all functionalities reserved to maintainer user profile are available and working correctly according to project documentation. |  |  |  |  |  |  |  |
| Ref. To Architecture |  | Architecture Reference |  |  |  |  |  |  |  |
| Reference Documents: |  | User manual HMI specification |  |  |  |  |  |  |  |
| Tools / Configuration |  | Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | Equipments (if applicable) Model Serial Number |  |  |  |  |  |  |  |
|  |  | Calibration Date (if applicable) |  |  |  |  |  |  |  |
| Prerequisites: - HW FAT successfully completed |  |  |  |  |  |  |  |  |  |
| Scenario: According to project documentation, check using the maintainer user profile, verify that each HMI function provided by the Self-diagnostic function is displayed correctly. |  |  |  |  |  |  |  |  |  |
| Location: Vehicle number OCC/ Control room |  |  |  |  |  |  |  |  |  |
| Attachments: If any |  |  |  |  |  |  |  |  |  |
| Test Method |  |  |  |  |  |  |  |  |  |
| Test Process (Action) |  |  | Pass / Fail Criteria (Check) |  |  |  |  |  | Pass / Fail result |
| P1 | Log in to the HMI using the "Maintainer" profile. |  | Confirm that the login is successful. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P2 | Navigate to the "Self-Diagnostic" of HMI. |  | Confirm that the "Reporting and Tracking Monitoring" HMI is displayed correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P3 | Navigate to the "Self-Diagnostic/ Reporting and Tracking Monitoring" of HMI. |  | Confirm that the "Self-Diagnostic/ Reporting and Tracking Monitoring " HMI is displayed correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P4 | Navigate to the "Self-Diagnostic/ Flow Data" of HMI. |  | Confirm that the "Self-Diagnostic/ Flow Data" HMI is displayed correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P5 | Navigate to the "Self-Diagnostic/ History RTM" of HMI. |  | Confirm that the " Self-Diagnostic/ History RTM" HMI is displayed correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P6 | Navigate to the "Self-Diagnostic/ History Flow data" of HMI. |  | Confirm that the " Self-Diagnostic/ History Flow data" HMI is displayed correctly. |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| Resources: i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| Date of Execution: |  |  |  |  |  |  |  |  |  |
| Tested by: |  |  |  |  | Witnessed By: |  |  |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  |
| Results: |  | □ PASS |  | □ FAIL |  | □ PASS WITH REMARKS |  |  |  |
| Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |

    - Test Case ID: APCS_FIT_021

| TEST CASE ID: APCS_FIT_021 |  | SUB-SYSTEM: APCS |  |  |  |  | Test Phase: FIT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Title: |  | Check APCS HMI functionalities for OCC Operator profile |  |  |  |  |  |  |  |
| Category |  | software |  |  |  |  |  |  |  |
| Ref. To Requirements: |  | C6-APCS-87 C6-APCS-3 C6-APCS-74 C6-APCS-6 C6-APCS-7 C6-APCS-8 C6-APCS-9 C6-APCS-10 C6-APCS-11 C6-APCS-12 |  |  |  |  |  |  |  |
| Acceptance Criteria |  | PASS / FAIL / PASS WITH COMMENTS / NOT APPLICABLE |  |  |  |  |  |  |  |
| Test Objective |  | Check that all functionalities, reserved to OCC Operator user profile and not tested by means of all other tests, are available and working correctly according to project documentation. |  |  |  |  |  |  |  |
| Ref. To Architecture |  | HMI figures …. |  |  |  |  |  |  |  |
| Reference Documents: |  | User manual HMI specification |  |  |  |  |  |  |  |
| Tools / Configuration |  | Software Version (if applicable) |  |  |  |  |  |  |  |
|  |  | Equipments (if applicable) OBCU Model Serial Number |  |  |  |  |  |  |  |
|  |  | Calibration Date (if applicable) |  |  |  |  |  |  |  |
| Prerequisites: - HW FAT successfully completed - Onboard MSN Network tests successfully completed |  |  |  |  |  |  |  |  |  |
| Scenario: According to project documentation, check using the maintainer user profile, verify that each HMI function provided by the main section including the buttons R.T.M and statistics are displayed correctly. |  |  |  |  |  |  |  |  |  |
| Location: Vehicle number OCC/ Control room |  |  |  |  |  |  |  |  |  |
| Attachments: If any |  |  |  |  |  |  |  |  |  |
| Test Method |  |  |  |  |  |  |  |  |  |
| Test Process (Action) |  |  | Pass / Fail Criteria (Check) |  |  |  |  |  | Pass / Fail result |
| P1 | [TL] The OCC operator login to APCS HMI iHMI as profile (train controller) |  | The OCC operator is logged in successfully |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P2 | [TL] Check the functionality of the R.T.M button that is highlighted in red when it’s clicked and is showing the vehicles and statistics buttons. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P3 | Check when the “vehicle button” is clicked that the vehicles in service are shown including the train ID, the last station this vehicle stopped at, the coloured diagnostic icon is visible with the colours green , red or yellow and also the icon of level of occupancy is visible with the colours white, green yellow orang or red. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P4 | Check when the stops icon is clicked, the list of stops’ names is visible and when a stop is selected, the details of the stop can be seen in the main section, the name of the stop is in bold and this stop is distinguished from the others by a red line on the left of the station name. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P5 | Check the data of last stop, Train ID, crowdedness of the whole train, the number of people and percentage of crowdedness for each carriage are visible in the topmost of the main section. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P6 | Select the M1 car from the main section and check if the people count, crowdedness for this car, people in and people out is visible in the train compoFITion section. |  |  |  |  |  |  |  |  |
| P7 | Select the R1 car from the main section and check if the people count, crowdedness for this car, people in and people out is visible in the train compoFITion section. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P8 | Select the S1 car from the main section and check if the people count, crowdedness for this car, people in and people out is visible in the train compoFITion section. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P9 | Select the M2 car from the main section and check if the people count, crowdedness for this car, people in and people out is visible in the train compoFITion section. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P10 | Check the people count and crowdedness data, for each station from the ‘stops detail’ menu in the main section is visible by clicking the ‘+’ button on each stop then check the graph of crowdedness and people count after pressing the toggle button. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P11 | Check when any stop is clicked from the stops icon , the station name, people in and out from the station , people in and out in specific time table are visible in the main section and when the station traffic toggle button is toggled , the graph for people in and out from the station in time table is visible in green and red colours. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P12 | Check in the congestions line graph that the green line and red lines which indicates the people in and out of the stations respectively are visible for each date , by hovering the cursor on these lines the tooltip providing detailed information on that specific time point is visualized and in a specific portion of graph can be zoomed in by using the zoom bar for the selected station form the station list, all when the statistics button has been chosen. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P13 | Check if the trend station by time dashboard is showing the graphical representation of all stations in green representing the people counting in the station and red representing the people out of the station are visible in the Dashboard Station Congestion Rate section when the statistics button is selected. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P14 | Check in the Dashboard Vehicle Ridership data visualization can be filtered by tracking ID, carriages and time filter when the statistics button is clicked. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P15 | Check if the inflow in red colour and outflow in green colour are visible for each carriage in the carriage congestion section in the Dashboard Vehicle Ridership when statistics button is clicked. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P16 | Check the table of congestion by station is listing the total numerical inflow and outflow data for each station related to the selected vehicle TID filter after choosing the statistics button and click the Dashboard Vehicle Ridership. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P17 | Check when any station in the table of congestion by station menu is clicked, the carriage congestion and congestion by station data for this station is visible, when statistics button is clicked, and Dashboard Vehicle Ridership has been chosen. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| P18 | Check if the user can export the data displayed in the dashboard in each format of PNG for static images of graphs and CSV for row data. |  |  |  |  |  |  |  | □ PASS □ FAIL □ PASS WITH REMARKS |
| Resources: i.e. Commissioning Manager, etc.. |  |  |  |  |  |  |  |  |  |
| TEST REPORT RESULT |  |  |  |  |  |  |  |  |  |
| Date of Execution: |  |  |  |  |  |  |  |  |  |
| Tested by: |  |  |  |  | Witnessed By: |  |  |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  |
| Results: |  | □ PASS |  | □ FAIL |  | □ PASS WITH REMARKS |  |  |  |
| Remarks and Resolution Notes: |  |  |  |  |  |  |  |  |  |